# Measurements

These are all given measurements.

| Type | Unit | Description |
| --- | --- | --- |
| Speed Over Ground | Km/h | Knots? (x 1,852 = km/h) |
| Depth | m | Depth is wrong. Draft is from keel to waterline (dependable on load). Depth is keel to deck (fixed). |
| Course Over Ground | Degrees (°) | The actual direction the vessel is moving relative to the earth's surface (GPS-based). |
| Heater 1 Fuel Rate | L/h | The volume of fuel flowing through the first heater per unit of time. |
| Heater 1 Fuel Temp | °C | The temperature of the fuel inside or exiting the first heater. |
| Heater 2 Fuel Rate | L/h | The volume of fuel flowing through the second heater per unit of time. |
| Heater 2 Fuel Temp | °C | The temperature of the fuel inside or exiting the second heater. |
| Generator FWD Fuel Rate | L/h | The fuel consumption rate of the forward (front) generator engine. |
| Generator FWD Real Power | kW | The actual electrical power produced by the forward generator used to perform work. |
| Generator FWD Apparent Power | kVA | The total power circulating in the forward generator's circuit (vector sum of real and reactive power). |
| Generator FWD Phase A Current | A | The electrical current flow in Phase A of the forward generator's three-phase output. |
| Generator FWD Phase B Current | A | The electrical current flow in Phase B of the forward generator's three-phase output. |
| Generator FWD Phase C Current | A | The electrical current flow in Phase C of the forward generator's three-phase output. |
| Generator Port AFT Fuel Rate | L/h | The fuel consumption rate of the aft generator located on the port (left) side. |
| Generator Port AFT Real Power | kW | The actual electrical power produced by the port aft generator. |
| Generator Port AFT Apparent Power | kVA | The total power circulating in the port aft generator's circuit. |
| Generator Port AFT Phase A Current | A | The electrical current flow in Phase A of the port aft generator. |
| Generator Port AFT Phase B Current | A | The electrical current flow in Phase B of the port aft generator. |
| Generator Port AFT Phase C Current | A | The electrical current flow in Phase C of the port aft generator. |
| Generator Starboard AFT Fuel Rate | L/h | The fuel consumption rate of the aft generator located on the starboard (right) side. |
| Generator Starboard AFT Real Power | kW | The actual electrical power produced by the starboard aft generator. |
| Generator Starboard AFT Apparent Power | kVA | The total power circulating in the starboard aft generator's circuit. |
| Generator Starboard AFT Phase A Current | A | The electrical current flow in Phase A of the starboard aft generator. |
| Generator Starboard AFT Phase B Current | A | The electrical current flow in Phase B of the starboard aft generator. |
| Generator Starboard AFT Phase C Current | A | The electrical current flow in Phase C of the starboard aft generator. |
| ME SB Alternator Voltage | V | The electrical voltage output of the alternator on the Starboard Main Engine. |
| ME SB Boost Pressure | Bar | The air pressure supplied by the turbocharger to the Starboard Main Engine's intake. |
| ME SB Coolant Pressure | Bar | The pressure within the cooling system of the Starboard Main Engine. |
| ME SB Coolant Temp | °C | The operating temperature of the coolant for the Starboard Main Engine. |
| ME SB Load | % | The percentage of the Starboard Main Engine's maximum power currently in use. |
| ME SB Exhaust Temp | °C | The temperature of the gases exiting the Starboard Main Engine's cylinders. |
| ME SB Fuel Rate | L/h | The hourly fuel consumption of the Starboard Main Engine. |
| ME SB Fuel Temp | °C | The temperature of the fuel entering the Starboard Main Engine. |
| ME SB Oil Pressure | Bar | The pressure of the lubricating oil within the Starboard Main Engine. |
| ME SB Oil Temp | °C | The temperature of the lubricating oil for the Starboard Main Engine. |
| ME SB RPM | Rev/min | The rotational speed of the Starboard Main Engine's crankshaft. |
| ME PS Alternator Voltage | V | The electrical voltage output of the alternator on the Port Side Main Engine. |
| ME PS Boost Pressure | Bar | The air pressure supplied by the turbocharger to the Port Side Main Engine's intake. |
| ME PS Coolant Pressure | Bar | The pressure within the cooling system of the Port Side Main Engine. |
| ME PS Coolant Temp | °C | The operating temperature of the coolant for the Port Side Main Engine. |
| ME PS Load | % | The percentage of the Port Side Main Engine's maximum power currently in use. |
| ME PS Exhaust Temp | °C | The temperature of the gases exiting the Port Side Main Engine's cylinders. |
| ME PS Fuel Rate | L/h | The hourly fuel consumption of the Port Side Main Engine. |
| ME PS Fuel Temp | °C | The temperature of the fuel entering the Port Side Main Engine. |
| ME PS Oil Pressure | Bar | The pressure of the lubricating oil within the Port Side Main Engine. |
| ME PS Oil Temp | °C | The temperature of the lubricating oil for the Port Side Main Engine. |
| ME PS RPM | Rev/min | The rotational speed of the Port Side Main Engine's crankshaft. |
| Bunker Aft Current Volume | L | The total volume of fuel currently stored in the aft (rear) bunker tank. |
| Bunker FWD Current Volume | L | The total volume of fuel currently stored in the forward (front) bunker tank. |
| Humidity Air | % | The relative humidity level of the ambient air. |
| Temperature Air | °C | The temperature of the surrounding ambient air. |
| App Wind Speed | Km/h | (knots?) Wind speed measured relative to the moving vessel. |
| App Wind Angle | Degrees (°) | The direction from which the wind hits the vessel relative to the bow. |